Verdict

Bro, do not submit this exact version yet. The topic is within Photonic Network Communications because the journal explicitly covers coherent optical systems, system/network performance modelling, network planning, and measurement/monitoring.

However, the present manuscript is likely to receive a desk rejection or major technical rejection. The main problem is not the Springer formatting; it is the mismatch among:

- the S+C+L claims;

- the limited C-band evidence;

- the accessible GitHub repository;

- the empirical residual correction;

- the absence of a network-level application.

I would rate the current submission readiness around 4/10. Both the code/evidence and paper writing need modification, but the code and experiments must be fixed first.

1. The most serious blocker: paper and GitHub do not currently match

Your manuscript reports a raw held-out bias of +2.784 dB, a corrected RMSE of 0.389 dB, and calibration/held-out wavelengths at 1535–1560 nm.

But the accessible repository’s newest validation commit is still labelled “preliminary C-band” and explicitly says that it does not support a full S+C+L publication claim. Its reported raw bias is approximately −1.612 dB, meaning the residual direction is opposite to the manuscript result.

The main configuration also still contains:

- calibration_status: UNVALIDATED_DEFAULTS;

- a warning that pump, amplifier, and Raman parameters are plausible defaults rather than measured values;

- a requirement to complete GNPy/SSFM validation before using the results as journal evidence.

Additionally, the code’s publication mode refuses to run while that status remains UNVALIDATED_DEFAULTS.

What to do

Create a dedicated release containing the exact code, configuration and data used for the uploaded manuscript:

releases/

└── pnc-v1.0/

├── config_pnc_final.yaml

├── gnpy/

│   ├── equipment.json

│   ├── topology_1span.json

│   ├── topology_4span.json

│   ├── topology_8span.json

│   └── spectral_information.json

├── validation_data/

│   ├── calibration_gnpy.csv

│   ├── heldout_gnpy.csv

│   └── ssfm_reference.csv

├── results/

├── figures/

├── MANIFEST.json

└── reproduce_paper.py

Then:

- create a GitHub release/tag such as pnc-submission-v1.0;

- record the exact commit SHA in the manuscript;

- archive the release in Zenodo and obtain a DOI;

- verify that a clean clone reproduces every table and figure;

- update the data-availability statement with the exact repository release and DOI.

Do not simply change UNVALIDATED_DEFAULTS to CALIBRATED. That field should change only after the calibration sources, parameter provenance and held-out results are committed.

2. The title currently overstates the evidence

The paper is titled as an S+C+L-band modelling framework, but all external validation is confined to approximately 1535–1560 nm, with held-out wavelengths around 1540–1555 nm. The manuscript itself acknowledges that this is only a local C-band validation region.

More importantly, the results section contains almost no actual S-, C-, and L-band performance evidence. It mainly presents the residual correction results.

Use this title with the present evidence

A Reproducible ISRS-Aware GSNR Estimator for S+C+L Optical Links with Held-Out C-Band GNPy Cross-Tool Verification

This makes it clear that:

- the model covers S+C+L;

- the independent cross-tool check is currently C-band only.

Better title after full validation

An ISRS-Aware QoT Estimator for S+C+L Optical Network Planning: Cross-Tool and SSFM Validation

The second title is much more suitable for Photonic Network Communications, especially after adding a network-planning experiment.

3. The manuscript does not explain the actual model sufficiently

The GitHub code contains a coupled Raman solver, wavelength-dependent fibre parameters, amplifier equalisation, ASE calculation, two NLI models, waveform processing and uncertainty analysis.

But the paper reduces the methodology to essentially one generic equation:

GSNR=PPASE+PNLI+PTRX.

That equation is standard and does not reveal the technical novelty of your framework. A reviewer cannot reconstruct or evaluate the actual model from the paper.

Add these equations and explanations

Your Methods section should include:

- Coupled signal-to-signal Raman equations

Explain the wavelength-dependent gain and depletion terms, photon-frequency ratio, attenuation, pump assumptions and numerical RK4 integration.

- Amplifier and ASE model

Define:

- per-band gain;

- noise figure;

- equivalent noise bandwidth;

- gain ripple or residual;

- amplifier gain limits;

- distributed Raman ASE treatment.

- ISRS-aware NLI calculation

Your PowerProfileGN implementation weights SCI and XCI using numerically generated longitudinal power profiles.

This may be a useful contribution, but you must provide:

- the mathematical derivation;

- assumptions;

- dimensions and units;

- how the longitudinal overlap factors are calculated;

- how the formula relates to or differs from Semrau’s model;

- the model’s validity limits.

- Multi-span accumulation

Explain the use of:

PNLI,N=N1+ϵPNLI,1.

The current implementation multiplies single-span ASE by the span count and scales single-span NLI using N1+ϵ.

A reviewer may challenge whether that approximation remains accurate when there are:

- wavelength-dependent amplifier gain limits;

- residual gain ripple;

- S/C/L amplifier differences;

- span-to-span power-profile changes.

Either implement recursive span-by-span propagation or benchmark the present approximation against it.

- GSNR-to-BER/EVM/GMI/NGMI mapping

Explain whether the mapping assumes AWGN, ideal DP-16QAM, Gray coding and independent polarizations. Separate analytical metrics from waveform-measured metrics.

- Calibration procedure

Clearly distinguish:

- physical-parameter calibration;

- receiver calibration;

- empirical residual correction;

- final held-out testing.

4. Replace the residual correction as the central contribution

At present, the raw held-out RMSE is 2.804 dB, and the main performance gain comes from subtracting a linear wavelength-dependent residual. The correction is fitted using only three unique calibration wavelengths.

Although there are nine rows, they represent three wavelengths repeated over three span counts. Therefore, the effective wavelength information is only three independent groups.

Table 4 also shows obvious span-dependent residual structure:

- approximately −0.25 dB for one span;

- approximately +0.41 to +0.48 dB for four and eight spans.

This means the wavelength-only correction is not fully representing the error mechanism.

Better approach: calibrate physical parameters

Fit a small set of physically meaningful parameters, for example:

θ=sαsNFsRamansγϵNLISα,λ.

These could represent:

- attenuation scale or slope;

- amplifier NF offset;

- Raman gain scale;

- nonlinear coefficient scale;

- multi-span coherence exponent;

- wavelength-dependent attenuation correction.

Use bounded nonlinear least squares:

from scipy.optimize import least_squares

def residuals(theta, calibration_cases):

predictions = run_model(theta, calibration_cases)

references = calibration_cases["gnpy_gsnr_db"].to_numpy()

uncertainties = calibration_cases["reference_uncertainty_db"].to_numpy()

return (predictions - references) / uncertainties

fit = least_squares(

residuals,

x0=initial_parameters,

bounds=(lower_bounds, upper_bounds),

args=(calibration_cases,),

)

Then freeze those parameters and evaluate untouched held-out cases.

If you retain the empirical correction

Call it:

post-model residual calibration

Do not present it as proof that the physical model itself agrees to 0.389 dB.

Also perform:

- leave-one-wavelength-out validation;

- leave-one-span-out validation;

- leave-one-launch-power-out validation;

- grouped bootstrap confidence intervals.

Row-level leave-one-out is insufficient because other rows from the same wavelength remain in training.

5. Expand validation beyond nine local held-out cases

The present validation tests only:

- three held-out wavelengths;

- three span counts;

- one flat launch condition;

- one simulator;

- one limited C-band region.

That is too small for a paper claiming an S+C+L framework.

Minimum publishable validation matrix

GNPy cross-tool cases

Use at least:

- S band: five wavelengths;

- C band: five wavelengths;

- L band: five wavelengths;

- span counts: 1, 4, 8 and 10;

- launch powers: −2, 0 and +2 dBm/channel;

- full and partial spectral loading;

- flat and shaped launch profiles.

You do not need every possible combination. A carefully designed set of 60–100 GNPy cases would be much stronger than the current 18 rows.

Independent SSFM cases

Add approximately 9–15 representative SSFM cases, for example:

- one edge and one central channel per band;

- 1-, 4- and 8-span links;

- ISRS enabled and disabled;

- representative neighbouring interferers or a manageable reduced WDM grid.

Compare:

- GSNR;

- NLI power;

- ASE power;

- received power tilt;

- error versus launch power.

GNPy and your framework may share GN-family assumptions, so GNPy is best described as cross-tool verification using an independent implementation, not fully independent physical validation.

Ablation experiments

Include:

- attenuation only;

- attenuation + amplifier ASE;

- ASE + conventional GN;

- ASE + ISRS-aware GN;

- ISRS-aware GN + transceiver noise;

- full calibrated model.

This will show which physical mechanisms create the observed residual.

6. Modify the multi-span code

In src/isrs_scl/link.py, the current calculation is effectively:

ase = n_spans * one_span_ase

nli = n_spans ** (1 + epsilon) * one_span_nli

gsnr = launch / (ase + nli + trx_noise)

That is computationally efficient, but for a broad S+C+L link you should implement or at least benchmark a recursive mode.

Suggested structure

def evaluate_recursive(self, launch_power_w, n_spans):

signal = launch_power_w.copy()

accumulated_ase = np.zeros_like(signal)

accumulated_nli = np.zeros_like(signal)

span_records = []

for span_index in range(n_spans):

span = self.span_model.evaluate(signal)

accumulated_ase += span.total_ase_psd_w_per_hz * receiver_bandwidth

accumulated_nli += span.nli.nli_power_w_per_span

signal = span.amplifier.output_powers_w

span_records.append({

"span": span_index + 1,

"input_power_w": signal.copy(),

"ase_w": accumulated_ase.copy(),

"nli_w": accumulated_nli.copy(),

})

total_noise = accumulated_ase + accumulated_nli + trx_noise

return signal / total_noise, span_records

Then compare:

- analytical scaling mode;

- recursive span mode;

- GNPy.

Report the maximum difference over wavelength and span count. If the difference is negligible, your present approximation becomes defensible.

7. Strengthen the Raman and amplifier input data

The default Raman gain curve is a synthetic multi-Gaussian approximation when no measured CSV is supplied. The code itself notes that journal work should replace it with measured fibre data.

Your configuration also uses assumed amplifier noise figures and pump powers.

For the publication configuration

Use:

raman:

gain_csv: data/raman_gain_measured_or_literature.csv

metadata:

calibration_status: CALIBRATED

calibration_sources:

- parameter: raman_gain_spectrum

file: data/raman_gain_measured_or_literature.csv

reference: "Full citation or vendor datasheet"

- parameter: amplifier_noise_figures

file: data/amplifier_nf_curves.csv

reference: "Full citation or vendor datasheet"

Every wavelength-dependent input should have:

- source;

- units;

- interpolation method;

- applicable wavelength range;

- uncertainty;

- version/hash.

Include a sensitivity analysis for:

- ±0.02 dB/km attenuation variation;

- ±0.3 dB NF variation;

- ±5–10% nonlinear coefficient;

- ±5–10% Raman gain;

- amplifier gain ripple.

Your code already contains uncertainty-analysis infrastructure, but the present manuscript does not report it adequately.

8. Add a network-level application for PNC

The journal is network-focused. Although physical-layer modelling is in scope, a pure link-calibration paper is less compelling than a paper showing how the model improves optical-network planning.

A recent PNC article using GSNR in C+L systems connected the physical model to a network reprovisioning problem and evaluated disruption and blocking in a representative topology.

Add one practical experiment

For example:

Evaluate the effect of ISRS-aware versus ISRS-unaware GSNR estimation on impairment-aware routing and modulation assignment.

Use a recognised topology such as NSFNET, COST239 or BT-UK and compare:

- accepted lightpath requests;

- blocking probability;

- selected modulation formats;

- total carried traffic;

- false-feasible lightpaths;

- false-infeasible lightpaths;

- average GSNR margin;

- runtime.

Compare three QoT estimators:

- conventional GN without ISRS;

- your raw ISRS-aware model;

- your calibrated ISRS-aware model.

This single addition would improve journal fit significantly.

9. Rewrite the Results section

The current Results section is almost completely about RMSE correction. For an S+C+L paper, include the following figures:

- S+C+L launch grid and channel count

- Power evolution along one span

- ISRS-induced spectral tilt with and without pumps

- ASE, NLI and transceiver-noise decomposition versus wavelength

- GSNR versus wavelength for 1, 4 and 8 spans

- Semrau model versus power-profile GN model

- GNPy residuals across all three bands

- SSFM comparison

- Uncertainty intervals

- Network-planning outcome

Move the current row-level Table 4 to supplementary material. Replace it in the main paper with a summary by:

- band;

- span count;

- launch power;

- loading condition.

10. Improve the writing and presentation

Remove these statements

The conclusion says that the results “support submission as a simulation-validated modelling paper.”

Remove this. A scientific conclusion should state what the evidence demonstrates, not judge whether the paper deserves submission.

Replace it with something like:

The results demonstrate that the calibrated estimator reproduces the selected GNPy cases within the reported uncertainty range while retaining channel-level interpretability. Broader cross-band and waveform-level validation remains necessary before generalising the accuracy to arbitrary S+C+L deployments.

Fix the contribution statement

The present contribution list focuses heavily on the validation workflow. Add the actual scientific novelty:

- coupled Raman power-profile modelling;

- power-profile-weighted SCI/XCI computation;

- receiver-aware QoT mapping;

- network-planning application.

Update the literature

The bibliography contains only 15 references and is dominated by foundational papers from 2012–2020. Add recent 2022–2026 work covering:

- S+C+L transmission;

- multiband optical-network planning;

- ISRS-aware GN/EGN models;

- GNPy developments;

- multiband amplifier models;

- SSFM validation;

- uncertainty-aware QoT estimation.

Aim for approximately 25–35 carefully selected references, not references added merely to increase the count.

11. Springer formatting corrections

Your current two-column template, numeric references, abstract length and six keywords are broadly aligned with the journal requirements. The journal asks for a 150–250-word abstract, 4–6 keywords, numbered references and LaTeX source using its two-column option. It also requires editable source files in addition to the compiled PDF.

However, fix these points:

- fom 06@pust.ac.bd appears to contain a space and may be an invalid email address.

- Add both authors’ ORCID identifiers.

- Remove chart titles from inside the figures. Springer asks authors not to put titles or captions inside illustrations.

- Increase figure lettering to a readable final size.

- Export all plots as vector PDF/EPS.

- Ensure the figures remain interpretable in grayscale.

- Use patterns or marker shapes in addition to colour.

- Put the exact GitHub release/DOI in the data-availability statement.

- Include a clear caption for Online Resource 1.

Recommended order of work

- Push the exact held-out validation code and results to GitHub.

- Create a frozen PNC release with a one-command reproduction script.

- Replace or benchmark the simple multi-span accumulation.

- Calibrate physical parameters instead of relying mainly on residual subtraction.

- Run grouped C/S/L GNPy validation.

- Add at least 9–15 SSFM cases.

- Add one network-planning application.

- Rewrite Methods and Results around the actual physics and network contribution.

- Revise the title, abstract, figures, references and data statement.

- Submit only after a clean clone reproduces every reported number.

The highest-priority issue is the accessible repository: the manuscript currently reports evidence that the main branch does not reproduce. Fixing that comes before any language polishing.